# Hybrid method for regional circular economy analysis: Stockholm County

## 1. Purpose and scope

This method provides the quantitative backbone for three of the five analytical dimensions requested by Region Stockholm:

- Dimension 1 — Resource use (tonnes). Material flows into, within, and out of the region, by sector and material category.

- Dimension 2 — Climate impact. Greenhouse gas emissions attributable to regional production and to regional consumption.

- Dimension 3 — Economic value loss (SEK). Quantification of value destroyed when materials exit the economy as waste rather than being recirculated.

The method combines a territorial production account (adapted from the Anthesis resource mapping method) with a regionally disaggregated multi-regional input-output model (EXIOBASE 3) to produce both production-based and consumption-based accounts. This dual perspective is essential for Stockholm, whose economy is service-dominated and import-heavy: the production account alone would understate the region's resource and climate footprint, while a consumption account alone would miss the material-intensive activity that does occur locally (primarily construction).

The method is designed to produce outputs that feed into the RISE project's other dimensions: the resilience analysis (Step 2B) can draw on the import dependency data from Dimension 1, and the governance roadmap (Step 2C) can draw on the sector prioritization from all three dimensions.

## 2. System boundaries

Geographic boundary. Stockholm County (Stockholms län, NUTS-3 code SE110), encompassing 26 municipalities and approximately 2.4 million inhabitants.

Temporal boundary. One calendar year. The base year should maximize overlap between available SNI economic data and the most recent EXIOBASE 3 release. As of early 2026, this is likely 2022.

Sectoral scope. All economic sectors are included in the EXIOBASE-based analysis (Dimensions 2 and 3 require full coverage of final demand). For the territorial production account (Dimension 1), sectors are prioritized by material throughput. For Stockholm, the key material-intensive sectors are: construction and civil engineering (SNI 41–43), food industry (SNI 10–12), chemical industry (SNI 20–21), metal products and machinery (SNI 24–28), printing and paper products (SNI 17–18), and waste management (SNI 38).

Material categories. Five categories following the Anthesis framework: biomass, metals, minerals (including chemicals), fossil, and recycled. EXIOBASE uses a slightly different categorization (Eurostat EW-MFA: biomass, metal ores, non-metallic minerals, fossil energy carriers). The concordance between these is addressed in Phase B.

Exclusions from the territorial account. Energy-related resource flows (electricity, heating), transport fuels, and water. These are excluded from Dimension 1 for consistency with the Anthesis method, but their climate impact is captured in Dimension 2 through EXIOBASE's GHG satellite accounts.

## 3. Method overview

The method consists of four phases.

Phase A (Steps 1–5) builds the territorial production account using the Anthesis method adapted for Stockholm. Output: tonnes of material inputs per sector within the region.

Phase B (Steps 6–9) disaggregates EXIOBASE into Stockholm and rest-of-Sweden, then computes consumption-based accounts for materials and GHG. Output: full material flow decomposition (in/within/out) and dual-perspective GHG accounts.

Phase C (Steps 10–11) quantifies economic value loss from linear material flows. Output: SEK value of materials lost to the economy through waste, by sector and material category.

Phase D (Steps 12–14) integrates the three dimensions, validates results, and produces the deliverables. Output: sector profiles, priority rankings, and workshop-ready materials.

## Phase A: Territorial production account

### Step 1: System boundaries for Stockholm

1.1. Confirm geographic boundary as Stockholm County.

1.2. Select base year (2024 recommended).

1.3. Identify all active economic sectors via SNI codes. Stockholm's economy differs structurally from the Anthesis pilot regions. Services, ICT, finance, and wholesale trade dominate by turnover, but construction, food, and chemicals dominate by material throughput.

1.4. Select priority sectors for the detailed territorial analysis. Minimum: construction, food, chemicals, metals/machinery, paper/printing, waste management. The service sectors (finance, ICT, professional services) are excluded from the production account but included in the consumption-based analysis in Phase B.

Data sources. SCB Statistikdatabasen (regional business statistics), Dun & Bradstreet (enterprise-level data), Region Stockholm (regional planning data).

### Step 2: Baseline data for Stockholm

2.1. Population, gross regional product (BRP), employment by sector.

2.2. Stockholm's natural resource context: limited primary resources, dominated by construction demand (population growth), food consumption (large consumer market), and logistics hub function.

2.3. Preliminary mapping of main physical resource types per included sector.

Data sources. SCB, Region Stockholm, SGU (mineral extraction), Länsstyrelsen Stockholm.

### Step 3: Sector-level economic data

3.1. For each selected sector at county level: number of enterprises, employees, turnover, raw materials, traded goods, COGS, personnel costs.

3.2. Identify the 3–5 largest enterprises per sector with five-digit SNI codes.

3.3. Same data at the national level for each sector.

3.4. Compute sector weights (critical for Phase B):

wsSthlm=Input goodssSthlmInput goodssSweden

These weights drive the EXIOBASE disaggregation.

Data sources. Dun & Bradstreet, SCB (custom data orders), Allabolag.se, company annual and sustainability reports.

### Step 4: Material composition data

Apply the Anthesis method's bottom-up or top-down approach per sector.

Recommended approach per sector for Stockholm:

- Construction (SNI 41–43): top-down. National data from Boverket, Mistra Carbon Exit (Karlsson et al. 2020), Wikells Sektionsdata. Scale to Stockholm using sector weights.

- Food (SNI 10–12): bottom-up where possible. Analyze 2–3 representative large companies (dairy, bakery, beverages).

- Chemicals (SNI 20–21): top-down. Fragmented sector. Use national data.

- Metals/machinery (SNI 24–28): bottom-up if feasible, otherwise top-down.

- Waste (SNI 38): direct data. Use Avfall Sverige county-level statistics.

See a prelimary mapping of key companies by sector in Section 8.

Data sources. Boverket Klimatdatabas, Wikells, UN Comtrade, Avfall Sverige, BASTA/Byggvarubedömningen, company reports, Länsstyrelsen environmental permits.

### Step 5: Calculate resource quantities

Apply Anthesis Equations 1–10 to produce the Phase A output.

Phase A output. Matrix [sectors × material categories] in tonnes of material inputs to production in Stockholm. Also: the sector weight vector wsSthlmfor Phase B.

## Phase B: EXIOBASE disaggregation and flow decomposition

This phase produces three things: (1) the full in/within/out material flow decomposition for Dimension 1, (2) the production-based and consumption-based GHG accounts for Dimension 2, and (3) the structural economic data needed for the value loss calculation in Phase C.

### Step 6: SNI-to-EXIOBASE concordance

6.1. Map each SNI two-digit code to corresponding EXIOBASE sector(s). Both derive from NACE Rev. 2, so the mapping is tractable.

6.2. Construct concordance matrix C [SNI sectors × EXIOBASE sectors]. For one-to-many mappings, estimate split ratios from national turnover data.

6.3. Include service sectors not covered in Phase A (finance, ICT, public administration) using national EXIOBASE weights scaled by Stockholm's employment share in those sectors. These are essential for the consumption-based accounts.

Data sources. EXIOBASE 3 documentation (Zenodo), Eurostat NACE concordance tables, SCB national accounts.

### Step 7: Disaggregate Sweden in EXIOBASE

This is the core technical step. Sweden's single entry in EXIOBASE is split into "Stockholm" and "rest-of-Sweden."

7.1. Load EXIOBASE 3 for the base year using pymrio.

7.2. For each EXIOBASE sector sector j, compute Stockholm's share:

αjSthlm=iCij×wiSthlm

7.3. Split Sweden's A-matrix column and row using αjshares. This creates a modified inter-industry matrix where Stockholm is a separate region.

7.4. Split Sweden's final demand vector (Y). Use regional household expenditure data from SCB where available. Fall back to income-weighted population share for categories where regional data is absent. Key splits:

- Household final demand: regional expenditure data or income-weighted population share (~24% of population, but higher income, so likely 28–30% of expenditure)

- Government final demand: regional government budget data from Region Stockholm

- Gross fixed capital formation: construction investment share from Step 3

7.5. Split satellite account rows (material extraction, GHG emissions) proportionally using αjshares.

Key assumption. Stockholm's inter-industry purchasing patterns mirror the national average per sector. This can be partially improved with regional import/trade data if available.

Computational note. The expanded system adds ~163 columns/rows. Total system size ~8,000+ sectors. Requires 16–32 GB RAM. The calc_all() operation may take 30–60 minutes.

Data sources. EXIOBASE 3 (Zenodo), SCB household expenditure surveys, SCB regional accounts, Region Stockholm budget data.

Other potential to do sanity check with Consumption Compass or other thing.

### Step 8: Compute all accounts

8.1. Calculate Leontief inverse on the modified system: L=(I-A)-1

8.2. Using pymrio's calc_all(), compute:

- S (direct satellite intensity per unit of output)

- M (total multiplier = S × L)

- D_pba (production-based account)

- D_cba (consumption-based account)

8.3. Extract Stockholm-specific results from both D_pba and D_cba for two satellite families:

- Material extraction satellites (for Dimension 1)

- GHG emission satellites (for Dimension 2)

### Step 9: Decompose material flows into in/within/out

This step transforms the EXIOBASE results into the flow decomposition that Region Stockholm has requested.

9.1. Flows IN (imports of embodied materials).

From the consumption-based account, identify the resource extraction that occurs outside Stockholm but is driven by Stockholm's final demand. This is extracted from D_cba by summing over all non-Stockholm rows for Stockholm's demand columns.

Flows INm=r≠SthlmD_cbar,mSthlm demand

This tells us: to satisfy Stockholm's consumption, how many tonnes of material m are extracted elsewhere in the world?

9.2. Flows WITHIN (internal circulation).

From the modified A-matrix, the Stockholm-to-Stockholm block represents inter-industry transactions within the region. Multiply by physical intensity coefficients (from Phase A) to convert to tonnes.

For the priority sectors identified in Phase A, compute the volume of materials that flow from one Stockholm sector to another. Focus on the largest internal flows by volume, economic value, and climate impact (as you specified).

Practical identification of priority "within" flows:

a) Extract the Stockholm-to-Stockholm submatrix from A. b) Rank sector pairs by transaction value. c) For the top 10–15 pairs, estimate physical material quantities using the price-per-weight keys from Phase A. d) Cross-reference with GHG multipliers to identify which internal flows carry the highest climate impact. e) Present the 5–8 most significant internal circulation flows.

9.3. Flows OUT (exports of embodied materials).

From the modified A-matrix and final demand structure, identify Stockholm's production that serves demand outside the region.

Flows OUTm=Total Stockholm productionm-Stockholm production serving Stockholm demandm

In EXIOBASE terms: Stockholm's total output minus the Stockholm-to-Stockholm intermediate demand and Stockholm final demand serviced by Stockholm production.

9.4. Material balance check.

Verify: Flows IN + Domestic extraction = Flows WITHIN (as throughput) + Flows OUT + Net additions to stock + Waste. This balance will not close perfectly due to the assumptions involved, but large imbalances signal data or assumption problems that need investigation.

Phase B output for Dimension 1. A Sankey-style flow decomposition showing material flows in, within, and out of Stockholm, by material category and priority sector.

Phase B output for Dimension 2. Two GHG accounts:

- Production-based: emissions from economic activity located in Stockholm

- Consumption-based: emissions worldwide driven by Stockholm's final demand

The difference reveals Stockholm's "carbon trade balance," the emissions embodied in net imports.

## Phase C: Economic value loss

### Step 10: Define the value loss framework

Value loss from use, wear, degradation and disposal is commonly treated as depreciation or deterioration (in the use phase) and as residual value when entering the disposal phase. One can attempt to evaluate both those in the economy (use phase) and those exiting the economy (disposal).

Depreciation can be defined as "the loss in value from any cause" and breaks it into "physical deterioration, functional obsolescence, and economic obsolescence.” For measuring it, the most common move is to separate the physical state of the asset or product from the value stream it can still generate. Hulten and Wykoff (1996) frame depreciation as a decline in financial value that tracks physical deterioration and retirement of assets, while Lund argues that an asset’s value is the "net economic value of its production over time" summarized by the net present value of present and future production. In practice, that means estimating remaining useful life, repair/maintenance costs, and expected service value, then discounting those future services back to today.

Once a product reaches end of life, the relevant question becomes recovered value rather than simple depreciation. Several papers treat reuse, remanufacture, repair, and recycling as the main ways to preserve embedded value, and one explicitly says these strategies are preferable to "direct disposal."(Wang and Tseng 2010) Another review of circular-economy methods says the goal is to maintain the "utility and value of material resources through product and component life extension, reuse, repair and remanufacturing, and material recycling." (T. Domenech, J. Stegemann 2021).

The concept of the value hill illustrates main concepts. Flow based accounting of value loss assuming a steady state (of both flows and depreciation/deterioration, ie. that it does not change over time). All waste outflow quantities are compared to material inflow values. For example, the value of product A waste outflow is assessed by assuming product A inflow value per quantity, thus capturing the entire loss of value between entry and exit from the economy. However, materials enter and exit the region at different stages of its value hill, based on what happens to it during its residence time in the region. Some materials enter as raw materials (at low part of the value hill) and exit as products at the highest point of the hill. Others enter as products and exit as raw materials for disposal or recycling.

Questions: do we (via Exiobase) capture the sum/difference based on all inflow/outflow values? Ie… production materials exit with more value than when they arrived while waste/residual materials exit with (or as) less value than when they arrived.

Or…. Do /should we focus on only the waste/residual values?

One yields an indication of how much value the region adds to materials vs how much it degrades. The latter only focuses on how much degradation occurs within the region. I believe the latter is more interesting, as it reflects a lost opportunity. Value enters but is not preserved. Some of this value is lost to natural causes (thermodynamics sets limits) but most is due to human, economic system effects. While we may accept a certain amount of degradation (via the laws of nature or via societal norms), we can certainly challenge the magnitude and norms that accept it.

So the practical toolkit is: track wear/deterioration and obsolescence during use; estimate depreciation via service-life or discounted-service methods; then estimate residual value at disposal through recovery options and end-of-life pathways.

The full value-added loss framing means we estimate not just the replacement cost of virgin material, but the cumulative processing value that is destroyed when materials exit the economy as waste instead of being recirculated. In this way, it is thought that we capture all value loss over time (both use phase depreciation AND loss at end-of-life disposal or recycling).

10.1. Conceptual definition. For each material stream that ends as waste:

Value lossm=Qmwaste×(Pmvirgin+VAmprocessing)
Where:

- Qmwaste= quantity of material mbecoming waste (tonnes), from Dimension 1 waste flow data

- Pmvirgin= market price of virgin material m(SEK/tonne)

- VAmprocessing= value added through processing stages before the material becomes waste (SEK/tonne)

The second term (VAmprocessing) is what distinguishes full value-added loss from simple material replacement cost. A tonne of construction steel that becomes demolition waste represents not just the ore price, but the smelting, rolling, fabrication, and installation value.

10.2. EXIOBASE provides the value-added structure. Each sector in EXIOBASE has a value-added row showing wages, profits, and taxes per unit of output. By tracing materials through the supply chain (using the Leontief inverse), we can compute the cumulative value added between extraction and the point where the material becomes waste.

Specifically, for material min sector s:

VAmprocessing=stagesvs×ls,extraction

Where vsis the value-added coefficient for sector sand ls,extractionis the Leontief coefficient linking extraction to the sector where waste occurs.

### Step 11: Compute value loss for Stockholm

11.1. Identify waste streams. From Dimension 1 (Phase A waste data from Avfall Sverige + Phase B flow decomposition), compile waste volumes by material category and originating sector for Stockholm County.

11.2. Assign virgin material prices. Use the same price-per-weight data collected in Phase A, Step 4 (UN Comtrade, Wikells).

11.3. Compute cumulative value added. From EXIOBASE's value-added satellite and the Leontief inverse, trace the processing chain for each material category. This gives the average value-added multiplier between extraction and final use.

For priority sectors, compute specific value-added chains. For example:

- Steel in construction: ore extraction → smelting → rolling → fabrication → installation → demolition waste. Each stage adds value that is lost when the material is landfilled or downcycled.

- Food biomass: agriculture → processing → packaging → retail → household waste. The value added at each stage is lost when food becomes waste.

11.4. Compute total value loss per sector and material category.

Total value loss=msQm,swaste×(Pmvirgin+VAm,sprocessing)

11.5. Compute the "circularity value recovery potential." For each waste stream, estimate what fraction could realistically be recovered through circular strategies (reuse, remanufacturing, recycling), and at what value retention rate. This creates a tiered picture:

- Reuse retains ~80–95% of processing value

- Remanufacturing retains ~50–70%

- Recycling retains ~20–40%

- Energy recovery retains ~5–10%

- Landfill retains 0%

Apply these retention rates to the waste stream composition to estimate the maximum recoverable value under different circular economy scenarios.

Phase C output. A matrix [sectors × material categories] showing:

- Total economic value lost (SEK) per year through linear waste flows

- Maximum recoverable value under optimistic circular scenarios

- Value recovery gap: the difference between current recovery and theoretical maximum

## Phase D: Integration and deliverables

### Step 12: Sector prioritization matrix

12.1. For each sector, compile all three dimensions:

- Material throughput (tonnes, from Dimension 1)

- GHG intensity, both production and consumption-based (tonnes CO2e, from Dimension 2)

- Economic value loss (SEK, from Dimension 3)

12.2. Rank sectors by each dimension. Identify sectors that score high on multiple dimensions (these are the priority targets for circular interventions).

12.3. For the top-priority sectors, develop detailed profiles showing: what materials flow through, where they come from, where they go, what emissions they cause, and what value is lost.

Expected priority sectors for Stockholm:

- Construction: likely dominant on Dimension 1 (material volume) and significant on Dimensions 2 and 3.

- Food: significant on all three dimensions due to Stockholm's large consumer base.

- Chemicals/plastics: potentially high on Dimension 2 (fossil-based GHG) and Dimension 3 (high processing value lost in waste).

### Step 13: Validation

13.1. Cross-check against independent sources: Boverket (construction), Jordbruksverket (food/agriculture), Avfall Sverige (waste), Naturvårdsverket (national emissions).

13.2. Sensitivity analysis on key assumptions: 40% COGS factor, final demand split, concordance mapping.

13.3. Present preliminary results to RISE team and Region Stockholm stakeholders for reality-checking.

### Step 14: Project deliverables

Aligned with the RISE project structure:

For the pre-study (Del 1, Steps 1A–1C):

- Method sketch for the workshop on April 27 (this document, refined)

- Data inventory: what is available, what needs to be procured, what gaps exist

- Preliminary estimates where simplified approaches can illustrate the method

For the main study (Del 2, Steps 2A–2B):

- Step 2A: Regional baseline = Dimensions 1 and 2 output (flow decomposition + dual GHG accounts)

- Step 2A: Materiality analysis = sector prioritization matrix from Step 12

- Step 2B: Socio-economic value analysis = Dimension 3 output (value loss quantification)

- Step 2B: Preparedness input = import dependency analysis from Dimension 1 outflows

Questions

What will this result be matched to or compared to? Some strategy or goals?

It will be comparable

How granual should it be?

Do things have to be recategorized to match Region Stockholms goals, ways of quantifying?

Socio-economic assessment- värdeläckage. Att definiera….

## 4. Alignment with the RISE project timeline

| RISE phase | Weeks | This method's contribution |
| --- | --- | --- |
| Step 1A: Calibration | 14–19 | Method sketch, workshop materials, scope confirmation |
| Step 1B: Method development | 17–25 | Phases A–C method specification, data inventory, concordance matrix, preliminary EXIOBASE test runs |
| Step 1C: Plan for main analysis | 23–27 | Computational pipeline specification, data procurement plan, delivery schedule |
| Step 2A: Regional baseline | 26–37 | Phase A execution (territorial account), Phase B execution (EXIOBASE disaggregation), flow decomposition, GHG accounts |
| Step 2B: Value analysis | 35–40 | Phase C execution (value loss), integration with resilience analysis |

Critical path items:

- Dun & Bradstreet data procurement must start by week 15–16 to be available for Phase A calculations by week 26.

- EXIOBASE download, parsing, and test disaggregation should happen during weeks 17–22 (Step 1B) so that computational issues are identified before the main study begins.

- The concordance matrix (Step 6) should be built and validated during the pre-study, not left to the main study phase.

## 5. Data requirements summary

| Data item | Source | Phase | Estimated cost/effort |
| --- | --- | --- | --- |
| Enterprise economic data, Stockholm County | Dun & Bradstreet | A | Commercial license |
| National sector data by SNI | SCB | A, B | Fee for custom orders |
| Population, BRP, employment | SCB | A, B | Free (Statistikdatabasen) |
| Material composition, representative firms | Company reports, interviews | A | Analyst time |
| National construction material data | Boverket, Karlsson et al. | A | Free |
| Material unit prices | UN Comtrade, Wikells | A, C | Wikells requires license |
| Waste statistics, Stockholm County | Avfall Sverige, SCB | A, C | Free/low cost |
| EXIOBASE 3 (A, Y, satellites) | Zenodo | B | Free |
| Regional household expenditure | SCB HUT survey | B | Fee for regional breakdown |
| Region Stockholm budget data | Region Stockholm | B | Request from project partner |
| Value-added coefficients | EXIOBASE (built-in) | C | Free (derived from model) |

## 6. Key assumptions and limitations

Assumption 1: 40% of COGS represents material inputs. Inherited from the Anthesis method. Test at 30% and 50% in sensitivity analysis.

Assumption 2: Stockholm's inter-industry linkages mirror the national structure. The proportional split of Sweden's A-matrix assumes homogeneous purchasing patterns. Stockholm may import more internationally and source less domestically. Partially addressable with regional trade data.

Assumption 3: Material composition is nationally homogeneous within a sector. Stockholm's construction mix may be more concrete-intensive (high-rise) and less timber-intensive than national average. Addressable through the bottom-up approach for key sectors.

Assumption 4: Value-added retention rates for circular strategies are based on literature averages. Actual retention depends on material condition, technology availability, and market demand. Present as scenario ranges, not point estimates.

Limitation: "Recycled" category has no direct EXIOBASE equivalent. The consumption-based account cannot directly quantify recycled flows. Dimension 1's territorial account (Phase A) is the primary source for recycling data.

Limitation: Within-flows are estimated, not measured. Internal circulation is derived from the model structure, not from observed physical transactions between Stockholm firms.

## 7. Outputs overview

| Dimension | Deliverable | Unit | Resolution |
| --- | --- | --- | --- |
| 1. Resource use | Flows IN to Stockholm | tonnes by material category | By sector and origin region |
| 1. Resource use | Flows WITHIN Stockholm | tonnes by material category | Top 5–8 sector-to-sector flows |
| 1. Resource use | Flows OUT from Stockholm | tonnes by material category | By sector and destination |
| 2. Climate impact | Production-based GHG | tonnes CO2e | By sector |
| 2. Climate impact | Consumption-based GHG | tonnes CO2e | By sector of final demand |
| 2. Climate impact | Carbon trade balance | tonnes CO2e | Net import/export |
| 3. Economic value | Value lost to waste | SEK | By sector and material |
| 3. Economic value | Recoverable value | SEK | By circular strategy tier |
| 3. Economic value | Value recovery gap | SEK | Current vs. theoretical max |
| Integrated | Sector prioritization | Multi-criteria ranking | Top sectors across all dimensions |

## 8. Preliminary mapping of key companies for bottom-up analysis

Below is a SEEA-MFA–compatible mapping and list of key companies that should be considered when analysing material consumption and production in Region Stockholm, structured by the material-intensive SNI sectors. Numbers are expressed as orders of magnitude and dominant material types rather than firm-specific confidential data. This is the level typically used in regional MFA, RMC/MF modelling, and EU CE indicator work, and it is consistent with how Eurostat/SCB MFA tables are downscaled.

Important methodological note
At regional level, company-specific tonnages are rarely observable. The standard approach is:

- assign firms to material categories,

- estimate sectoral intensities (t/EUR or t/activity unit),

- allocate flows spatially (production sites, projects, population).

The figures below therefore represent typical annual ranges for major Stockholm actors, not audited company data.

1. Construction and Civil Engineering

SNI 41–43 – very high material throughput (concrete, steel, aggregates, asphalt)

These firms have major operations, sites, or headquarters in Region Stockholm and are thought to represent a large part of primary construction material inflows in the region (cement, aggregates, steel).

- Skanska AB – large-scale construction, infrastructure, and civil works; major consumer of concrete, steel, asphalt

- NCC AB (Solna) – buildings, transport infrastructure, asphalt and aggregates

- Peab AB – housing, civil engineering, and construction materials

- JM AB (Stockholm) – residential construction, high volume of concrete and building materials

- Infranord AB (Solna) – rail and transport infrastructure, steel and ballast intensive

SEEA role: DMC (minerals & metals), Net Additions to Stock (NAS)

Dominant material categories

| Material | Typical regional order of magnitude |
| --- | --- |
| Concrete (ready-mix, precast) | 1–5 million t/yr |
| Aggregates (sand, gravel, crushed rock) | 3–10 million t/yr |
| Asphalt | 0.5–1.5 million t/yr |
| Steel (reinforcement, structural) | 150–400 kt/yr |
| Excavated soils & rock | 5–15 million t/yr (gross movements) |

Company linkage

- Skanska / NCC / Peab → concrete, aggregates, asphalt, rebar

- JM → concrete, aggregates (residential stock)

- Infranord → steel (rails), ballast, concrete

MFA treatment

- Counted mainly in DMC – non-metallic minerals

- Appears as NAS (buildings & infrastructure)

- Circularity captured via secondary aggregates and recycled steel

2. Food Industry

SNI 10–12 – biomass, packaging, energy-intensive processing

Key food and beverage producers/distributors in Region Stockholm that are thougt to have significant material stocks and flows of biomass, packaging, and waste streams.:

- The Absolut Company AB (Stockholm) – alcohol production, glass bottles, agricultural inputs [companydata.com]

- Scan Sverige AB (Stockholm) – meat processing, feed, packaging, by-products [companydata.com]

- Spendrups Bryggeri AB (Stockholm) – beverages, glass, aluminium cans [companydata.com]

- Arvid Nordquist AB (Järfälla) – coffee and food products, packaging materials [companydata.com]

- Vitamin Well AB (Stockholm) – bottled drinks, PET/plastic packaging flows [zoominfo.com]

SEEA role: Biomass DMC, short residence time

Dominant material categories

| Material | Typical regional order of magnitude |
| --- | --- |
| Agricultural biomass (raw food input) | 250–600 kt/yr |
| Water (process & product) | 5–15 Mm³/yr |
| Packaging – glass | 80–150 kt/yr |
| Packaging – PET & plastics | 30–70 kt/yr |
| Packaging – aluminium | 10–25 kt/yr |
| Food waste & by-products | 60–150 kt/yr |

Company linkage

- Absolut / Spendrups → glass bottles, water, barley/grain

- Scan → livestock biomass, bones, fat by-products

- Vitamin Well → PET bottles

- Arvid Nordquist → coffee biomass, paper packaging

MFA treatment

- Appears in DMC – biomass

- No long-term stock formation

- Circularity via deposit systems, biogas, animal feed

3. Chemical and Pharmaceutical Industry

SNI 20–21 – high value, high environmental relevance materials

Key producers and facilities in or near Stockholm that use and generate large amounts of heavily regulated chemical inputs, solvents, water use, and hazardous waste.:

- AstraZeneca AB (Södertälje) – pharmaceuticals, chemical intermediates, solvents [pharmchoices.com], [companydata.com]

- Octapharma AB (Stockholm) – biopharmaceutical production, plasma-derived materials [companydata.com]

- APL – Apotek Produktion & Laboratorier AB (Huddinge) – pharmaceutical manufacturing and compounding [companydata.com]

- Recipharm AB (Jordbro) – contract pharmaceutical manufacturing (CDMO) [ensun.io]

- Cederroth / Orkla Health (Stockholm region) – consumer chemicals and hygiene products [pharmchoices.com]

3. Chemical & Pharmaceutical Industry (SNI 20–21)

Companies: AstraZeneca (Södertälje), Recipharm, Octapharma, APL
SEEA role: Imported intermediates, hazardous outputs

Dominant material categories

| Material | Typical regional order of magnitude |
| --- | --- |
| Chemical intermediates & APIs | 10–40 kt/yr |
| Solvents (ethanol, acetone, etc.) | 20–80 kt/yr (mostly recycled internally) |
| Water | 3–8 Mm³/yr |
| Hazardous waste | 5–15 kt/yr |

Company linkage

- AstraZeneca → solvents, APIs, water

- Octapharma → plasma-derived biological materials

- Recipharm / APL → formulation chemicals

MFA treatment

- Low impact on DMC by mass

- High relevance for hazardous waste indicators

- Circularity mainly through closed-loop solvents

4. Metal Products and Machinery

SNI 24–28 – steel, aluminium, machinery and components

Key Stockholm-based or Stockholm-operating companies that use and generate large amounts of primary and secondary metal flows, machining waste, and imported material stocks.:

- Epiroc Sweden AB (Norsborg) – mining and construction machinery, steel-intensive products [dnb.com]

- Scania Industrial Maintenance AB (Södertälje) – heavy machinery maintenance, metal parts [dnb.com]

- Toyota Material Handling Sweden AB (Solna) – forklifts, logistics machinery [dnb.com]

- Atlas Copco AB (Stockholm HQ) – industrial machinery, compressed air systems [f6s.com]

- System 3R International AB (Vällingby) – precision metal tooling and fixtures [dnb.com]

SEEA role: Metal DMC, long-life capital stock

Dominant material categories

| Material | Typical regional order of magnitude |
| --- | --- |
| Steel & iron | 120–300 kt/yr |
| Aluminium | 25–60 kt/yr |
| Copper & alloys | 5–12 kt/yr |
| Industrial scrap (recyclable) | 80–200 kt/yr |

Company linkage

- Epiroc / Atlas Copco → alloy steel, precision metals

- Toyota Material Handling → steel frames, batteries

- Scania maintenance → replacement parts, scrap metals

MFA treatment

- Appears in DMC – metals

- Large contribution to productive NAS (machinery)

- Strongly boosts Circular Material Use (secondary metals)

5. Printing and Paper Products

SNI 17–18 – high fiber, water, and energy input

Key companies linked to Region Stockholm material flows of biomass, fiber recovery, and packaging:

- Essity AB (Stockholm HQ) – tissue and hygiene products, fiber-based materials [largestcompanies.com]

- Billerud AB (Stockholm HQ) – packaging paper and board [largestcompanies.com]

- Holmen AB (Stockholm HQ) – paper and packaging materials [largestcompanies.com]

- Re:NewCell AB (Stockholm) – textile-to-cellulose recycling (emerging circular flow) [dnb.com]

- Iggesund Paperboard AB (Stockholm sales presence) – paperboard products [dnb.com]

SEEA role: Biomass DMC, short–medium lifetimes

Dominant material categories

| Material | Typical regional order of magnitude |
| --- | --- |
| Pulp & paper products | 400–900 kt/yr |
| Recovered paper (inputs) | 250–600 kt/yr |
| Textile cellulose feedstock | 30–60 kt/yr |
| Process water | 10–25 Mm³/yr |

Company linkage

- Essity → tissue & hygiene products

- Billerud / Holmen → packaging board

- Re:NewCell → textile-to-cellulose recycling

MFA treatment

- Appears in DMC – biomass

- Strong influence on CMU rate

- Fast cycling materials (low NAS)

6. Waste Management

SNI 38 – end-of-life material flows and circular economy

Key system operators in Region Stockholm for secondary material recovery, losses, and circularity in the region.:

- Stockholm Vatten och Avfall AB – municipal waste, recycling, and treatment [ensun.io], [start.stockholm]

- REMONDIS Recycling Sweden AB (Spånga) – industrial and commercial waste streams [remondisrecycling.se], [remondisrecycling.se]

- Envac AB (Stockholm) – automated waste collection systems [ensun.io]

- Stena Recycling Stockholm – metals, paper, plastics recycling (regional operations) [folotop.com]

- Tysslinge Åkeri AB / Recycling units (Södertälje area) – construction and bulk waste flows [scrapmonster.com]

SEEA role: Secondary material flows (not DMC drivers)

Dominant handled material categories

| Material | Typical regional order of magnitude |
| --- | --- |
| Municipal solid waste | 900–1,100 kt/yr |
| C&D waste | 1,500–3,000 kt/yr |
| Metal scrap | 300–500 kt/yr |
| Biowaste | 150–250 kt/yr |
| WtE residues (slag, ash) | 200–400 kt/yr |

MFA treatment

- Impacts recycling rates, CMU, landfill rate

- Does not reduce DMC directly, only redirects outputs

SEEA-MFA and CE significance.